	                                                                                                                                                                                                                                                                                                                                                                                                                                                 SHIP PARTICULARS m/v AFINA

	Name (Ex-Name):
	Afina (LMZ Trodos, Katerina S, Sea Merchant)

	Built:
	Sept 1983, Japan

	Yard:


	Shimizu Works Nippon Kokan K.K., Shimizu, Japan, Builder 

Building ID 397

	Owners:
	Mindaugas Shipping Limited

	Managers (ISM):
	CSM Energy, Cyprus

	Flag:
	LIBERIA

	Call Sign:
	5LHK5

	Port Of Registry:
	Monrovia

	Oficial No./ IMO No.:
	22152 / 8029272

	Class Register No:
	8329668

	INMARSAT-C No:
	1-463737368; 2-463737369; 3-463737367

	Fleet:
	Iridium - 881677759713

	Mobile phone No:
	n/a

	MMSI:
	636022152

	Type:
	Bulk Carrier

	Master and Crew:
	Ukrainian, Indonesian

	ABS Class:
	A1, Bulk Carrier, AMS, ACCU

	P & I:
	The North of England Protecting & Indemnity Association Ltd.

	H & M / Value:
	The North of England Protecting & Indemnity Association Ltd

	International GRT/NRT:
	21.168 / 12.980

	Suez (fitted):
	21,639.92 / 18,134.28

	Panama (fitted):
	22,498.60 / 18,263.88

	LOA / LBP / Breadth Moulded /
Depth:
	177.50m / 167.35m / 29.51m /14.8m

	Draft in ballast condition (50 pct of stores):
	Fore: 4.30 m; Aft: 6.70 m /  TPC – 40.9


	Air draft max
	45,80 m


Light Ship Weight – 8316.16 mt
	
	Draft
	DWAT
	TPC

	Summer SW (draft/dwt/tpc)
	10.665 m
	34.708 mt
	44.8

	Winter SW
	10.443 m
	34.427 mt
	44.6

	Tropical SW
	10.887 m
	36.419 mt
	45.0

	Fresh
	10.905 m
	35.421 mt
	45.0

	Tropical Fresh
	11.127 m
	36.397 mt
	45.2

	

	Draft, m 
	Sea Water DWT, mt 
	Fresh Water DWT, mt 
	TPC 

	5.0 
	11,357 
	
	40.6 

	6.0 
	15,447 
	
	41.2 

	7.0 
	19,600 
	18,940 
	41.7 

	7.5 
	21,690 
	20,990 
	42.1 

	8.0 
	23,800 
	23,010 
	42.4 

	8.5 
	25,920 
	25,100 
	42.8 

	9.0 
	28,080 
	27,210 
	43.2 

	9.5 
	30,640 
	29,700 
	43.8 

	10.0 
	32,490 
	31,490 
	44.2 

	10.5 
	34,700 
	33,640 
	44.6 


	Fresh Water Allowance:
	240 mm


	Number of holds / hatches:
	5 / 5

	Hold
	Length, m
	Width, m
	Height, m
	Tank top (LxW), m

	1
	19.00
	19.00 x 29.30
	13.00
	20.00
	7.00/21.00

	2
	25.00
	29.50
	13.00
	22.50
	21.20

	3
	26.00
	29.50
	13.00
	23.60
	21.30

	4
	27.50
	29.40
	13.00
	25.50
	21.20

	5
	26.50
	29.40 x 22.50
	13.00
	27.00
	21.20/13.20


	Hold
	Grain
	Bale

	1
	5.383 cubm 
	190.099 cubf
	5.193 cubm
	183.389 cubf

	2
	9.179 cubm
	324.153 cubf
	8.900 cubm
	314.300 cubf

	3
	8.719 cubm
	307.909 cubf
	8.435 cubm
	297.880 cubf

	4
	9.129 cubm
	322.388 cubf
	8.835 cubm
	312.005 cubf

	5
	9.048 cubm
	319.527 cubf
	8.832 cubm
	311.900 cubf 

	Total
	41.458 cubm
	1.464.086 cubf
	40.195 cubm
	1.419.473 cubf


	Vsl’s holds are free of obstructions

	Tanktops are steel and suitable for grabs discharging.

	Holds are fitted with Australian type ladders.

No CO2 in holds and no smoke detection system.

	Ventilation in holds:
	Natural

	Hatch covers type:
	Folding type

	Size of hatch No 1:
	13.015m x 10.30m

	Hatches No 2 – 5:
	19.55m x 15.10m

	Distance from waterline to the top of hatch coaming in ballast / laden:
	11.8 (aft part) / 6.5 m

	Distance from keel to top of hatch coaming:
	16.53 m

	Distance from ships rail to edge of hatch covers/coaming:
	7.00 m

	Tank top strength:
	Hold No. 1, 3 – 23.1 mts per sqm
Hold No. 2, 4 – 15.6 mts per sqm

Hold No. 5 – 17.6 mts per sqm

	Hatchcovers strength:
	1.75 mts per sqm

	Upper Deck strength:
	3.45 mts per sqm

	Distance from keel to the top of the mast:
	45.8 m

	Two cement holes per hold.


	Vessel deballasting rate:
	Upto 350 mts per hour as max

	Max acceptable loading rate:
	Upto 500 mts per hour as max

	Max ballast capacity:
	12.500 mts ssw

	Fresh water capacity:
	230 mts

	Fresh water daily consumption:
	10 mts

	Constant excluding fresh water
	250 mts

	Vessel is fitted to carriage of grain in accordance with Chapter IV of SOLAS 1974 and amendments without requiring bagging strapping and securing when loading a full cargo (deadweight) of heavy grain in bulk (sf 42 cbft) with ends untrimmed.

	Number of holds which may be left slack without requiring bagging, strapping and securing:
	ONE


	Vessel has 2 canes SWL – 35 mts (12.0 – 30.0 m) . SWL – 30 mts (12.0 – 36.0 m) lifting capacity.

	Make and type:
	Liebherr, Germany, Electro hydraulic jib crane. 


	Full bunker capacities (IFO/MGO):
	IFO 1428 cub.m / MGO 434 cub.m

	Unpumpables: 
	IFO 80 mts / MGO 10 mts

	Specification of fuel (IFO/MGO):
	IFO 180 RME 25 SG MAX 0.988 / MGO DMA MAX SULPHUR 0.2 TO ISO 8217:2005 (E)

	Daily consumption at sea at 12.5 knts speed basis Beaufort force 3 Douglas sea state 2:
	IFO – 25.0 mts / MGO – 2.5 mts

	Daily consumption at port (idle/working):
	MGO – 2.0/3.5-4.0 mts; IFO – 1.1 mts

	Vessel burning MGO in M/E while maneuvering/navigating in shallow waters, canals, rivers, in/out of ports and shifting between berth.

	Service speed:
	12.5 knots

	Main engine:
	Mitsubishi Sulzer 6RLB66

	Output / RPM:
	10.660 bhp / 131 rpm

	Propeller Diam x Pitch
	5500 mm x 3715 mm


Master m/v “AFINA”
